Read me to start the air bearing table

Matlab preparation:

- Start Matlab

- Load “Inertia.mat” file. In DCMotorCharacterization, load “motorResponse1.mat”, “motorResponse2.mat”, “motorResponse3.mat”, “motorResponse4.mat”.

- Open “commandABT.slx” file

- Go to Hardware Settings / Hardware Implementation / Target hardware resources / WiFi properties, enter the properties of your WiFi hotspot. In External mode, set XCP on WiFi.

- Connect the computer to the hotspot.

Table preparation:

- Check the tension level of the batteries (>12V)

- Chek also the connection between the batteries and the pins.

- Connect the Arduino alimentation pin: black to the ground and red to the VIN pin

- Connect the Arduino board to the computer

- Set the command angles

Click on “Run on hardware to lunch the table”

- Once “Connecting to MKR1000” is displayed, disconnect the Arduino board from the computer

- Maintain the table during the wheel initialization

- Release when the wheels are initialized (the initial speeds are in the Simulink program, in the “controller” block.